MFD FIRE MARSHAL’S OFFICE

1500 W. Wall St Midland, TX 79701

432-685-7333

MOBILE FOOD VENDORS

Please have all of the following items completed, prior to inspection.

- All LP-Gas containers (empty or full) shall be secured properly in the upright position on the outside of the vehicle.

- All gas lines are free of leaks at all fittings and appliances.

- All vendors shall carry a (3A-40BC minimum) Fire Extinguisher. In addition, a Type K, for grease fire, portable extinguisher shall also be carried when cooking operations produce grease laden vapors. All extinguishers shall have approved current annual inspection date tags. If possible, all extinguishers shall be mounted. Vendors which do not cook shall have a minimum 2A-10BC extinguisher only.

- If equipped, all commercial kitchen fire suppression systems shall have approved current licensed inspections as required by the adopted IFC.

- Ventilation hoods shall be kept clean as needed or at the discretion of the Fire Inspector.

- Refueling of generators shall be performed in an approved location not less than 20 feet from the Mobile Food Unit. All fuel shall be stored in a UL or FM listed flammable liquid safety container and in an approved location.

- All 120v electrical wiring shall be installed and maintained according to the National Electrical Code.

- Open wiring and junction boxes are prohibited.

- Overall appearance of the vehicle exterior shall be maintained in good condition

- Vehicle and trailer wiring, lights and signals shall meet DOT standards.